# Micro Nozzle Design for Cube-Sats

Version 0 – explaining the processes [work in progress]

# Executive Summary

[To be completed]

Contents

Executive Summary	1

1.	Introduction	2

1.1.	Purpose	2

1.2.	Design Scope and Hypothesis	2

1.3.	Background/Previous research	2

2.	Methodology	3

2.1.	Design Approach	3

2.2.	Assumptions	3

2.3.	Computational Process and Key Equations	4

2.4.	Software Implementation and Workflow	6

2.4.1.	Mode 1: Single Calculation	6

2.4.2.	Mode 2: Parameter Sweep	6

3.	Results & Discussions	8

3.1.	Mode 1: Baseline Design Analysis	8

3.2.	Mode 2: Parameter Sweep Analysis	11

3.3.	Discussion of Sweep Results (Key Findings)	13

4.	Conclusion	14

5.	Reference	15

6.	Appendices	16

6.1.	Previous calculation notes	16

# Introduction

## Purpose

To serve as a computational aid for verifying calculations from one of the PhD papers, particularly for applications, such as micro plasma thrusters or cold gas systems where isentropic flow models are a relevant starting point.

## Design Scope and Hypothesis

This study focuses on the design of a nozzle intended to operate in vacuum conditions, utilising Argon as the propellant in a cold gas thruster system. The following hypotheses are investigated:

- Stagnation temperature (T0​) will be the primary driver of specific impulse (Isp), a key measure of propellant efficiency.

- Nozzle throat radius (R throat ​) will primarily scale the thrust level and overall nozzle dimensions (such as bell nozzle length).

- For the specified low ambient pressure (vacuum conditions), optimal nozzle performance can be achieved without the formation of detrimental internal shock waves across a practical range of operating stagnation temperatures and throat sizes

## Background/Previous research

[To be completed]

# Methodology

## Design Approach

The methodology involves two main stages:

- Baseline Nozzle Design (Mode 1): Determination of key dimensions and performance characteristics for a reference set of operating conditions. This includes calculating the optimal exit area ratio (AeAt ​) for expansion to the specified ambient pressure and verifying that no internal shock waves are present under these conditions.

- Parametric Study (Mode 2): Investigation of the impact of varying key input parameters on nozzle performance and geometry. This study focuses on:

  - Stagnation temperature (T0​)

  - Nozzle throat radius (R throat​)

## Assumptions

The analysis relies on the following fundamental assumptions:

- Fluid Properties and Flow Model:

  - Propellant: Argon (Ar)

  - Specific Heat Ratio (γ): 1.667

  - Molar Mass (M): 0.039948 kg/mol

  - Universal Gas Constant (Ru​): 8.314 J/(mol·K)

  - Flow Model: One-dimensional, steady, compressible, isentropic flow of a calorically perfect gas.

- Operating Conditions (Baseline for Mode 1; T0​ varied in Mode 2):

  - Stagnation Pressure (P0​): 666.612 Pa

  - Stagnation Temperature (T0​): 300 K (baseline)

  - Ambient Pressure (P ambient​): 0.01333 Pa (representative of vacuum)

- Geometric Inputs (Baseline for Mode 1; R throat​ varied in Mode 2):

  - Throat Radius (R throat​): 0.005 m (baseline)

  - Chamber Radius (R chamber​): 0.08 m

  - Converging Section Half-Angle (θ c​): 60 degrees

  - Diverging Section Half-Angle (for conical performance calculation, θ d​): 15 degrees

  - Bell Nozzle Type: 80% Rao optimized parabolic contour

### Computational Process and Key Equations

The core calculations rely on standard one-dimensional isentropic flow theory for a perfect gas. The process to determine baseline performance and geometry (Mode 1) is as follows:

- Optimal Expansion Conditions: Given P0, T0, γ, specific gas constant (Rs=RuM), and Pambient​, the `nozzle_calculator` module determines the exit Mach number (Me​) and corresponding exit area ratio (AeAt​) that results in the nozzle exit pressure (Pe​) closely matching Pambient​ (within a defined PRESSURE_TOLERANCE). This is achieved by solving the following key isentropic relations:

  - Area Mach Number Relation: AAthroat=γ2-γ+12γ-1×1+γ+12M2γ+12γ-1M

  - Using Ideal Gas Equation pv=RρT and Ideal compressible gas equation m=AP0T0 γRM1+γ+12M2-γ+12γ-1, we can derive the area ratio to get 2 following equation

    - Pressure Ratio: PP0=1+γ-12M2-γγ-1

    - Temperature ratio: TT0=1+γ-12M2-1

- Conical Nozzle Performance: Using the determined AeAt​ and Me​, performance metrics (thrust, Isp, mass flow rate, etc.) are calculated. For the simplicity, the length of the diverging section (Ldiv​) for a conical nozzle with the specified half-angle (θd​) is calculated using Ldiv=Re-Rttanθd .

- Normal Shock Check: The `shock_calculator` module evaluates if a normal shock wave would exist inside the nozzle for the calculated AeAt and the actual back pressure ratio (PambientP0 ​). This ensures the nozzle operates as intended in the vacuum environment (i.e., overexpanded or ideally expanded, without detrimental internal shocks).

- Bell Nozzle Geometry: If the shock check is satisfactory (no internal shock), I then calculated the nozzle performance.

m=Athroat×P0×γRspecific×T0×2γ+1γ+1γ-1

vexit=2γγ-1 Rspecific T0 (1-PP0γ-1γ)

Thrust=m×vexit×ΔPexit,ambient×Aexit

CF=ThrustP0×Athroat

C*=P0×Athroatm=IspgCF=γRspecificT0γ 2γ+1γ+1γ-1

Isp=Thrustm×g

- Then, the bell_nozzle_module calculates the contour of an 80% Rao optimised parabolic bell nozzle. This uses the AeAt and Rt​ and is based on empirical data and geometric constructions involving circular arcs and a quadratic Bezier curve. The contour points are exported for potential CAD use

- Calculating heat – power required

Pheater=mcP(T0-Tinitial)

## Software Implementation and Workflow

The analysis is performed using a custom Python script (main_nozzle_analysis.py) that orchestrates calls to three core modules within the nozzle_lib package: nozzle_calculator.py, shock_calculator.py, and bell_nozzle_module.py. The main script operates in two modes, selectable by the user at runtime:

### Mode 1: Single Calculation

- Performs a detailed analysis for a single set of globally defined input parameters.

- Outputs comprehensive performance data, shock analysis results, and bell nozzle geometry (if applicable).

- Generates plots for nozzle flow characteristics and the bell nozzle contour (2D/3D), which are saved to the output_plots/ directory. Bell nozzle contour data is saved to a CSV file in the output_csv/ directory.

### Mode 2: Parameter Sweep

The idea is to conduct a parametric study by iterating over user-defined ranges of stagnation temperature (T0​) and throat radius (Rthroat​). The objectives are:

- To assess the impact of T0​ (potentially varied by a heater in the stagnation chamber) on propulsion efficiency (Isp) and other performance metrics.

- To investigate how varying R throat influences thrust, mass flow rate, and overall nozzle dimensions (e.g., bell nozzle length), which is crucial for minimizing size and weight in applications like CubeSats.

For each combination, calculations are performed similarly to Mode 1 (though detailed plotting for each iteration is typically disabled for speed). Results from all iterations are collected, summarized in the console, saved to sweep_results.csv, and key trends are plotted and saved to output_plots/.

# Results & Discussions

## Mode 1: Baseline Design Analysis

Inputs: T0​=300K, Rthroat​=5mm, P0​=666.612Pa, Pamb​=0.01333Pa, γ=1.667

- Nozzle Design Calculations (Conical Performance Approximation):

  - Target ambient pressure (0.01333 Pa) matched within tolerance

  - Calculated Exit Mach (Me​): 7.000

  - Calculated Exit Area Ratio (AeAt ​): 24.11316

  - Calculated Exit Pressure (Pe​): 0.53344 Pa

  - Diverging Section Length for 15° Conical Nozzle (Ldiv​): 72.97 mm

  - Bell Nozzle Contour (80% Rao) Length: 58.38 mm

  - Weight: 93.64 g. (Aluminium Alloy 6061)

- Normal Shock Location Analysis:

  - Status: Flow is overexpanded (isentropic supersonic flow, or oblique shocks may form outside the nozzle).

  - Calculated Pressure Boundaries (Pb​/P0​):

    - Subsonic Exit: 0.99857

    - Normal Shock at Exit: 0.04882

    - Supersonic Isentropic Exit (target Pe​=Pambient​): 0.00080

  - Actual Back Pressure Ratio (Pambient​/P0​): 1.9997×10−5

  - Conclusion: Normal shock check passed; no internal shock predicted.

- Calculated Performance:

  - Exit Velocity (ve​): 542.30 m/s

  - Mass Flow Rate (m): 0.00015 kg/s

  - Thrust (F): 0.08351 N

  - Thrust Coefficient (CF​): 1.59497

  - Characteristic Velocity (c*): 344.07 m/s

  - Specific Impulse (Isp​): 56.00 s

- Nozzle Characteristics Plots (Isentropic Flow):

Figure 1: Isentropic flow parameters (Pressure Ratio, Temperature Ratio, Area Ratio vs. Mach Number) and axial pressure profile for the baseline nozzle design (T0=300K, R throat=5mm)

- Bell Nozzle Contour Plots:

Figure 2: 2D contour and 3D representation of the 80% Rao bell nozzle for Ae/At=24.113 and Rthroat=5.00 mm

- CAD Renders

Figure 3: Nozzle model

Figure 4: Nozzle model - cross section

## Mode 2: Parameter Sweep Analysis

- Sweep Ranges:

  - Stagnation Temperature (T0​): 300 K to 1000 K (5 steps)

  - Nozzle Throat Radius (Rthroat​): 1.0 mm to 6.0 mm (6 steps)

  - Total Iterations: 30

- Fixed Parameters during Sweep (derived for optimal expansion):

  - Exit Area Ratio (AeAt​): ~66.19 (determined for the fixed P0Pambiant​ and γ, and thus remains constant for optimal expansion across the sweep)

An excerpt summary of the sweep results is presented in Table 1.

Table 1: Summary of key performance metrics from the parameter sweep over T0 & Rthroat

| Iter | T0 (K) | R_thr (mm) | Isp (s) | Thrust (N) | Ln (mm) | Shock? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 300 | 1.0 | 56.46 | 0.0034 | 21.3 | False |
| 2 | 475 | 1.0 | 71.04 | 0.0034 | 21.3 | False |
| 3 | 650 | 1.0 | 83.11 | 0.0034 | 21.3 | False |
| 4 | 825 | 1.0 | 93.63 | 0.0034 | 21.3 | False |
| 5 | 1000 | 1.0 | 103.08 | 0.0034 | 21.3 | False |
| 6 | 300 | 2.0 | 56.46 | 0.0135 | 42.6 | False |
| 8 | 650 | 2.0 | 83.11 | 0.0135 | 42.6 | False |
| 9 | 825 | 2.0 | 93.63 | 0.0135 | 42.6 | False |
| 10 | 1000 | 2.0 | 103.08 | 0.0135 | 42.6 | False |
| 11 | 300 | 3.0 | 56.46 | 0.0303 | 63.9 | False |
| 12 | 475 | 3.0 | 71.04 | 0.0303 | 63.9 | False |
| 13 | 650 | 3.0 | 83.11 | 0.0303 | 63.9 | False |
| 14 | 825 | 3.0 | 93.63 | 0.0303 | 63.9 | False |
| 15 | 1000 | 3.0 | 103.08 | 0.0303 | 63.9 | False |
| 16 | 300 | 4.0 | 56.46 | 0.0539 | 85.2 | False |
| 17 | 475 | 4.0 | 71.04 | 0.0539 | 85.2 | False |
| 18 | 650 | 4.0 | 83.11 | 0.0539 | 85.2 | False |
| 19 | 825 | 4.0 | 93.63 | 0.0539 | 85.2 | False |
| 20 | 1000 | 4.0 | 103.08 | 0.0539 | 85.2 | False |
| 21 | 300 | 5.0 | 56.46 | 0.0842 | 106.5 | False |
| 22 | 475 | 5.0 | 71.04 | 0.0842 | 106.5 | False |
| 23 | 650 | 5.0 | 83.11 | 0.0842 | 106.5 | False |
| 24 | 825 | 5.0 | 93.63 | 0.0842 | 106.5 | False |
| 25 | 1000 | 5.0 | 103.08 | 0.0842 | 106.5 | False |
| 26 | 300 | 6.0 | 56.46 | 0.1212 | 127.8 | False |
| 27 | 475 | 6.0 | 71.04 | 0.1212 | 127.8 | False |
| 28 | 650 | 6.0 | 83.11 | 0.1212 | 127.8 | False |
| 29 | 825 | 6.0 | 93.63 | 0.1212 | 127.8 | False |
| 30 | 1000 | 6.0 | 103.08 | 0.1212 | 127.8 | False |

Key trends observed from the parameter sweep are illustrated below.

Figure 5: Performance data from sweep parameters case

What we learnt:

## Discussion of Sweep Results (Key Findings)

[To be updated]

- Specific Impulse (Isp): Isp, a measure of propellant efficiency, showed a strong dependence on stagnation temperature (T0​) but was independent of throat radius (R throat​). Isp increased from approximately 56.5 s at 300 K to 103.1 s at 1000 K. This follows the theoretical relationship where Isp∝T0

- Thrust: Thrust depends on both T0​ and R throat​​.

- For a fixed R throat​​, thrust was observed to be nearly independent of T0​. This is consistent with theoretical expectations for ideal expansion to a fixed ambient pressure where Thrust=P0 At CF ​. Since P0​ is constant, At​ is constant for a fixed R throat​​, and the thrust coefficient (CF​) is primarily dependent on the (constant) Ae​/At​ and γ, thrust shows minimal variation with T0​.

- For a fixed T0​, thrust increases significantly and proportionally with the throat area (At=πRthroat2​) due to the corresponding increase in mass flow rate.

- Example Thrust Range (from Rt​=1mm to Rt​=6mm, values constant across T0​ range):

  - Rt​=1mm: ~0.0034 N

  - Rt​=6mm: ~0.1212 N

- Mass Flow Rate: Mass flow rate is directly proportional to throat area (At​) and inversely proportional to the square root of stagnation temperature (m∝P0AtT0​). The plots (bottom left of Figure 5) demonstrate this scaling, with distinct lines for each throat radius and a decreasing trend with increasing T0​.

- Bell Nozzle Length (Ln): The calculated length of the 80% Rao bell nozzle scales directly with the throat radius (R throat​) because the entire contour geometry is scaled proportionally for a fixed Ae​/At​. It ranged from approximately 21.3 mm (Rt​=1mm) to 127.8 mm (Rt​=6mm) and was independent of T0​.

- Normal Shock Check: No internal normal shocks were predicted for any of the conditions tested within the sweep range. The calculated exit pressure consistently aimed to match the low ambient pressure, indicating operation in the ideally expanded or overexpanded regime (relative to atmosphere, but optimally expanded to the target Pambient​).

# Conclusion

[To be updated]

The computational analysis highlights the expected trade-offs in Argon cold gas thruster nozzle design. Stagnation temperature (T0​) is confirmed as the primary driver for propellant efficiency (Isp), offering significant gains with increased temperature. The throat radius (R throat​) directly scales the thrust level and the overall physical size (including length) of the nozzle.

For the given operating conditions (expansion into vacuum), a consistent optimal area ratio (Ae​/At​≈66.19) was determined, and no internal shock formation was predicted, validating the suitability of the design approach for this environment. The choice of operating parameters for a micro-nozzle will involve balancing desired thrust level (scaled by throat area​), efficiency (driven by T0​), and physical constraints on size and mass (influenced by R throat and LN​).

This Python toolkit serves as an effective aid for exploring these preliminary design considerations. Future work could involve integrating a nozzle mass model to directly assess weight implications and exploring more complex flow phenomena if operating conditions deviate significantly from ideal assumptions.

# Reference

- Sutton, G. P., & Biblarz, O. (2016). Rocket Propulsion Elements (9th ed.). John Wiley & Sons, Inc.

- Richards, G. (n.d.). The thrust optimised parabolic nozzle. Aspirespace. Retrieved May 16, 2025, from http://www.aspirespace.org.uk/downloads/Thrust%20optimised%20parabolic%20nozzle.pdf

- TheEngineer, J. (n.d.). Normal Shock Calculations. JoshTheEngineer.com. Retrieved May 16, 2025, from http://www.joshtheengineer.com/
(Note: It's best to link to a specific article on normal shocks if one was predominantly used. If the logic was adapted from general principles or multiple examples on the site, a general reference is acceptable. I've given a placeholder title "Normal Shock Calculations".)

# Appendices

## Previous calculation notes

Equations

Athroat=mPthroatR*TthroatM*γ  _____________________ (1)

Pthroat=Pchamber1+γ-12-γk-1____________________ (2)

Tthroat=Tchamber1-γ-12 ______________________________ (3)

Substitute (2) & (3) into (1)

Athroat=mPchamber1+γ-12-γγ-1R*TchamberM*γ*(1-γ-12)

Or Plug  Values in≫ Athroat=m324.7015R*450.1125M*γ

∴Athroat=0.023086918m